Section 083613 
iNSULATED sTEEL-bACK Sectional Doors
Model 470

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward acting sectional steel doors.

Related Requirements

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 087100 - Door Hardware>>:  Lock cylinders.

- ~~~~~ MASTER NOTE ~~~~~~
This cross-reference is applicable only to doors with electric motor operation.
~~~ END OF MASTER NOTE ~~~~

Section << 260533.13 - Conduit for Electrical Systems>>:  Conduit from electric circuit to operator and from operator to control station, and empty conduit from control units to door operator.

Section << 260583 - Wiring Connections>>.

Reference Standards

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

UL 325 - Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems; Current Edition, Including All Revisions.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Installation Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Warranty

Manufacturer Warranty:  Provide manufacturer warranty for counterbalance shaft assembly for years indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

PART 2 PRODUCTS

Manufacturer

~~~~~ MASTER NOTE ~~~~~~
  The Overhead Door™ Brand is one of the most recognized and respected brands in the garage door industry. C. G. Johnson invented the upward-lifting garage door in 1921 and the electric garage door opener in 1926. For over 100 years, the Overhead Door™ Brand has stood for product quality and expertise, as well as professional service. From commercial and industrial upward-lifting doors and access systems to residential garage doors and openers, we combine quality, dependable products with reliable service to consistently give home and business owners confidence and peace of mind. To learn more about what our company and our distributors have accomplished over this century of greatness you can visit https://www.overheaddoor100.com/. The Overhead Door™ Brand is a garage door brand of Overhead Door Corporation, a Sanwa Holdings Company. Headquarters are located in Lewisville, TX. With our nationwide network of more than 450 authorized Ribbon Distributors, we are a leading provider of garage door systems, and we continue to lead the way with reliable solutions and unmatched professional installation, service and support that keeps customers coming back.   Contact Information: Overhead Door 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: 1 (800) 929-3667 (DOOR) Email: info@overheaddoor.com   Contact for Design Professional: Technical Support Group Phone: 1-800-929-4692 Email: info@overheaddoor.com   Online CEU Courses   Request Lunch & Learn   https://www.overheaddoor.com/home   Plant Location: Lewistown, Pennsylvania; Williamsport, Pennsylvania; Pensacola, Florida; Grand Island, Nebraska    
~~~ END OF MASTER NOTE ~~~~

Overhead Door Corporation; www.overheaddoor.com; 1 (800) 929-3667.

Substitutions: Not permitted.

UPWARD ACTING SECTIONAL STEEL DOORS

Insulated Steel-Back Sectional Doors:  Overhead Door Corporation; Model 470.

Panels:  Steel construction with exterior sheet, interior sheet, and core reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 14 feet 1 inch (4.3 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door width for wind testing is 18 feet (5.5 m).
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand uniform pressure of << _____ psf (_____ kPa)>>.

Wind Load:  Not required.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:

Exterior Steel Thickness:  << 25 gauge, 0.018 inch (0.46 mm)>> galvanized steel.

Panel Thickness:  << 2 inches (51 mm)>>.

Joint Profile:  Tongue and groove.

Section Height:  Manufacturer's standard based on door height.

Insulation:  Bonded polystyrene.

Panel R-value (RSI-value):  << 9.83 (1.73)>>.

Interior Backer:  Painted galvanized steel, << 30 gauge, 0.011 inch (0.28 mm)>>.

Exterior Surface Profile:  Ribbed.

Lites:  << Not required;  Double thermal acrylic;  Insulated double strength billet (DSB);  Clear short; or  Clear long>>.

- ~~~~~ MASTER NOTE ~~~~~~
Doors with 16.75-inch (4.3 m) sections available only in white and almond.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White;  Almond;  Brown;  Sandstone; or  Desert Tan>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Finish:  Galvanized steel.

End Stiles:  << 20 gauge, 0.032 inch (0.81 mm)>> galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

UPWARD ACTING Sectional Aluminum Doors

Aluminum Sectional Doors:  Overhead Door Corporation; << Model 511;  Model 521; or  Model 522>>.

Panels:  Aluminum construction and reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Model 511: 16 feet 2 inches (6.2 m).
Model 521: 26 feet 2 inches (8 m).
Model 522: 18 feet 2 inches (5.5 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights: 
Model 511: 16 feet 1 inch (4.9 m).
Model 521: 20 feet 1 inch (6.1 m).
Model 522: 14 feet 1 inch (4.3 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
U-factor values:
Model 521 with double strength billet (DSB) clear, tempered, or obscure glazing: 0.30.
Model 521 with DSB Low-E coated glazing: 0.28.
~~~ END OF MASTER NOTE ~~~~

U-Factor:  << Not required;  0.30; or  0.28>>.

- ~~~~~ MASTER NOTE ~~~~~~
R-value (RSI-value) for Model 521:
With double strength billet (DSB) clear, tempered, or obscure glazing IGUs: R-value (RSI-value): 2.87 (0.51).
With clear polycarbonate IGUs: R-value (RSI-value): 2.93 (0.52).
With solar bronze IGUs: R-value (RSI-value): 3.17 (0.56).
With Low-E coating IGUs: R-value (RSI-value): 3.43 (0.60).
With SolarBan 70XL argon-filled IGUs: R-value (RSI-value): 4.09 (0.72).
With 1/4-inch thick multi-wall polycarbonate: R-value (RSI-value): 2.75 (0.48).
With 3/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.21 (0.57).
With 5/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.48 (0.61).
With 3/8-inch extruded polystyrene-filled solid panels: R-value (RSI-value): 2.60 (0.46).
~~~ END OF MASTER NOTE ~~~~

R-value (RSI-value):  << 2.60 (0.46);  2.75 (0.48);  2.87 (0.51);  2.93 (0.52);  3.17 (0.56);  3.21 (0.57);  3.43 (0.60);  3.48 (0.61); or  4.09 (0.72)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:  Extruded 6063-T6 aluminum.

- ~~~~~ MASTER NOTE ~~~~~~
Models 511 and 521: 1-3/4 inches (45 mm).
Model 522: 1-3/8 inches (35 mm). 
~~~ END OF MASTER NOTE ~~~~

Panel Thickness:  << 1-3/8 inches (35 mm); or  1-3/4 inches (45 mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 511 and 521 have shiplap joint profile.
Model 522 has tongue and groove joint profile.
~~~ END OF MASTER NOTE ~~~~

Joint Profile:  << Tongue and groove; or  Shiplap>>.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
Glazing options available per model:
Models 511 and 521: 1/8 inch (3 mm) DSB glass, 1/8 inch (3 mm) and 1/4 inch (6 mm) tempered DSB glass, 1/8 inch (3mm) and 1/4 inch (6 mm) acrylic, 1/8 inch (3 mm) and 1/4 inch (6 mm) clear polycarbonate, 1/4 inch (6 mm) wired glass.
Insulated DSB vision lites: 1/2 inch (13 mm) DSB glass, 1/2 inch (13 mm) tempered DSB glass, 1/2 inch (13 mm) acrylic, 1/2 inch (13 mm) polycarbonate.

Model 522: 1/4 inch (6 mm) opaque white, opaque black, mirrored gray, mirrored bronze, translucent black. DSB: Double strength billet.
~~~ END OF MASTER NOTE ~~~~

Glazing:  << Not required;  1/8 inch (3 mm) DSB glass;  1/8 inch (3 mm) tempered DSB glass;  1/4 inch (6 mm) tempered DSB glass;  1/8 inch (3 mm) acrylic;  1/4 inch (6 mm) acrylic;  1/8 inch (3 mm) clear polycarbonate;  1/4 inch (6 mm) clear polycarbonate;  1/4 inch (6 mm) wired glass;  1/2 inch (13 mm) DSB glass;  1/2 inch (13 mm) tempered DSB glass;  1/2 inch (13 mm) acrylic;  1/2 inch (13 mm) polycarbonate;  1/4 inch (6 mm) opaque white;  1/4 inch (6 mm) opaque black;  1/4 inch (6 mm) mirrored gray;  1/4 inch (6 mm) mirrored bronze; or  1/4 inch (6 mm) translucent black>>.

- ~~~~~ MASTER NOTE ~~~~~~
Finish options per model:
Models 511 and 521: Clear anodized, light bronze anodized, medium bronze anodized, dark bronze anodized, black anodized, powder coat.
Model 522:  Dark bronze anodized, black anodized, powder coated white, powder coated black, powder coated bronze.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << Clear anodized;  Light bronze anodized;  Medium bronze anodized;  Dark bronze anodized;  Black anodized; or  Powder coat; color: _____>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

Aluminum Sectional Doors:  Overhead Door Corporation; Model 521S.

Panels:  Aluminum construction and reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width: 20 feet 2 inches (6.2 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height: 18 feet 1 inch (5.5 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at speed of << 15.7 inches (399 mm)>> per second.

Closing Speed:  Door to operate at speed of << 10.4 inches (264 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
U-factor values:
With DSB clear, tempered, or obscure glazing: 0.30.
With DSB Low-E coated glazing: 0.28. DSB: Double strength billet.
~~~ END OF MASTER NOTE ~~~~

U-Factor:  << 0.30; or  0.28>>.

- ~~~~~ MASTER NOTE ~~~~~~
R-value (RSI-value) for Model 521S:
With double strength billet (DSB) clear, tempered, or obscure glazing IGUs: R-value (RSI-value): 2.87 (0.51).
With clear polycarbonate IGUs: R-value (RSI-value): 2.93 (0.52).
With solar bronze IGUs: R-value (RSI-value): 3.17 (0.56).
With Low-E coating IGUs: R-value (RSI-value): 3.43 (0.60).
With SolarBan 70XL argon-filled IGUs: R-value (RSI-value): 4.09 (0.72).
With 1/4-inch thick multi-wall polycarbonate: R-value (RSI-value): 2.75 (0.48).
With 3/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.21 (0.57).
With 5/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.48 (0.61).
With 3/8-inch extruded polystyrene-filled solid panels: R-value (RSI-value): 2.60 (0.46).
~~~ END OF MASTER NOTE ~~~~

R-value (RSI-value):  << 2.60 (0.46);  2.75 (0.48);  2.87 (0.51);  2.93 (0.52);  3.17 (0.56);  3.21 (0.57);  3.43 (0.60);  3.48 (0.61); or  4.09 (0.72)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum 300,000 cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:  Extruded 6063-T6 aluminum.

Panel Thickness:  << 1-3/4 inches (45 mm)>>.

Joint Profile:  Shiplap.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
DSB: Double strength billet.
~~~ END OF MASTER NOTE ~~~~

Glazing:  << Not required;  1/8 inch (3 mm) DSB glass;  1/8 inch (3 mm) tempered DSB glass;  1/4 inch (6 mm) tempered DSB glass;  1/8 inch (3 mm) acrylic;  1/4 inch (6 mm) acrylic;  1/8 inch (3 mm) clear polycarbonate;  1/4 inch (6 mm) clear polycarbonate;  1/4 inch (6 mm) wired glass;  1/2 inch (13 mm) DSB glass;  1/2 inch (13 mm) tempered DSB glass;  1/2 inch (13 mm) acrylic; or  1/2 inch (13 mm) polycarbonate>>.

Exterior Finish:  << Clear anodized;  Light bronze anodized;  Medium bronze anodized;  Dark bronze anodized;  Black anodized; or  Powder coat; color: _____>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Manual Operation:  Chain Hoist.

Motor Operation:  Provide UL-listed direct drive, integrated gear motor and brake electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second. Provide operator and drive assembly with all wiring harnesses needed from manufacturer.

Timer to Close:  Automatic closing controlled by adjustable hold-open time delay.

Operation Supply Voltage:  << 208V 1 phase 60 Hz, 190 to 207V range;  208V 3 phase 60 Hz, 190 to 207V range;  230V 1 phase 60 Hz, 208 to 245V range;  230V 3 phase 60 Hz, 208 to 245V range;  460V 3 phase 60 Hz, 456 to 495V range; or  575V 3 phase 60 Hz>>.

Control Panel:  << Medium BTR Control; or  Full BTR Control>> Electronic controller with microprocessor self-diagnostics. Digital readout indicates door action, alarm conditions and fault conditions.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  LED Lights>>.

Actuation Device:  Provide << push button;  key switch;  vehicle detection;  loop detector;  motion detector;  treadle switch;  radio control;  door timer; and  photocell operation>>.

Motor Mounting:  Shaft mount.

Obstruction Safety Detection:

Primary, Monitored:

Safety Edge:  << Wireless electric sensing edge; or  2-wire sensing edge>> complying with <<UL 325>>, 2010 edition.

Secondary, Non-Monitored:

Photoelectric Sensors:  << Reflective;  Dual; or  _______>>.

PART 3 EXECUTION

Examination

Verify wall openings are ready to receive work and opening dimensions and tolerances are within specified limits.

Verify electric power is available and meets manufacturer's requirements.

Preparation

Prepare opening to permit correct installation of door unit to perimeter air and vapor barrier seal.

Apply primer to wood frame.

Installation

Install door unit assembly in accordance with manufacturer's instructions.

Anchor assembly to wall construction and building framing without distortion or stress.

Securely brace door tracks suspended from structure. Secure tracks to structural members only.

Fit and align door assembly including hardware.

Coordinate installation of electrical service. Complete power and control wiring from disconnect to unit components.

Install << perimeter trim;  closures; and  ___________>>.

Tolerances

Maximum Variation from Plumb:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch in 10 feet (3 mm in 3 m); or  _____ inch in 10 feet (_____ mm in 3 m)>> straight edge.

Maintain dimensional tolerances and alignment with adjacent work.

Adjusting

Adjust door assembly for smooth operation and full contact with weatherstripping.

Confirm operation is in accordance with manufacturer's instructions with present field representative. Identify adjustments to door assembly.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean doors and frames<<  and glazing;    _____; or  None - N/A>>.

Remove<<  temporary;    _____; or  None - N/A>> labels and visible markings.

Protection

Protect installed products from damage until Date of Substantial Completion.

Do not permit construction traffic through overhead door openings after adjustment and cleaning.

END OF SECTION