Section 083323
upward Coiling Doors
Model 653 Allura Shutter

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward coiling security shutters.

Related Requirements

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 083313 - Coiling Counter Doors>>:  Additional types of overhead coiling doors.

Section << 087100 - Door Hardware>>:  Cylinder cores and keys.

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Reference Standards

ASHRAE Std 90.1 I-P - Energy Standard for Buildings Except Low-Rise Residential Buildings; Most Recent Edition Adopted by Authority Having Jurisdiction, Including All Applicable Amendments and Supplements.

ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process; 2023.

ASTM E330/E330M - Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference; 2014 (Reapproved 2021).

UL (DIR) - Online Certifications Directory; Current Edition.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Quality Assurance

Manufacturer Qualifications:  Company specializing in performing work of this section with minimum of << 5 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience in fabrication and installation of security closures.

Installer Qualifications:  Company specializing in performing work of this section with minimum of << 3 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience and approved by manufacturer.

Products Requiring Electrical Connection:  Listed and classified by << UL (DIR);  TUV;  testing firm acceptable to authorities having jurisdiction; or  _____>> as suitable for purpose specified.

Delivery, Storage, and Handling

Store products in manufacturer's unopened packaging until ready for installation.

Protect materials from exposure to moisture.

Store materials in dry, warm, ventilated, weathertight location.

Warranty

Manufacturer Door-Only Warranty:  Provide manufacturer warranty for door assembly for duration indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

PART 2 PRODUCTS

Manufacturer

Overhead Door Corporation; www.overheaddoor.com; 1 (800) 929-3667.

Substitutions: Not permitted.

Upward Coiling security shutters

Overhead DoorTM Brand; Model 653.

Slats:  Interlocking, extruded aluminum slats.

~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 14 feet (4.3 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  << Surface-mounted on side indicated on drawings; or  Between jamb>>.

Opening Speed:  Door to operate at variable speed up to << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 12 inches (305 mm)>> per second.

Curtain Material:  << 0.050-inch (1.27 mm)>> thick aluminum.

Fenestrated Slats:  << Not required;  Open fenestration; 1-by-1-inch (25.4 by 25.4 mm) uniformly spaced openings over full curtain;  Partially open fenestration; 1-by-1-inch (25.4 by 25.4 mm) uniformly spaced openings. Height of fenestrated curtain segment as indicated on drawings;  Perforated slats; full curtain; or  Perforated slats; partial curtain. Height of perforated curtain segment as indicated on drawings>>.

Aluminum Finish:  << White;  Almond;  Silver powder coat to match look of clear anodized aluminum;  Bronze powder coat to match look of bronze anodized; or  PowderGuard Premium powder coat; _____ color>>.

Bottom Bar:  6063-T6 aluminum.

Profile:  << Compact extruded; or  Extruded tube>>.

Finish:  Match curtain.

- ~~~~~ MASTER NOTE ~~~~~~
Slide lock and padlockable slide lock available only with compact extruded bottom bar. May be placed left, right, or at both jambs.
Cylinder and Best cylinder only available in center location on extruded tube bottom bar.
~~~ END OF MASTER NOTE ~~~~

Locking Options to Include:  << None;  Slide lock;  Cylinder; or  Best cylinder>>.

Lock Locations:  << Both jambs;  Left only;  Right only; or  Center>>.

Lock Access:  << Coil side;  Opposite coil side; or  Both sides>>.

Interlock micro switch on locking doors with tube motor operation.

Weatherstripping and Seals:

Bottom Seal Astragal:  Required.

Side Guides, Channels:  Constructed of aluminum extruded channels with continuous PVC wear strips.

Finish:  Match curtain.

Brackets:  Galvanized steel to support counterbalance, curtain, and hood.

Finish:  Match curtain.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (1:400 mm/sec)>> of span. Adjustable spring tension required.

Hood:  Aluminum. Box sized to match manufacturer's recommendation based on door height. Provide with intermediate support brackets as required on drawings.

Finish:  << Match curtain; or  PowderGuard Premium powder coat; _____ color>>.

Manual Operation:  << Push-up; or  Crank>>.

- ~~~~~ MASTER NOTE ~~~~~~
Motor only available on door width equal to or minimum 3 feet (914 mm) and maximum 14 feet (4.3 m), with height maximum 6 feet (1.8 m).
~~~ END OF MASTER NOTE ~~~~

Motor Operation:  Provide UL-listed electric tube motor; size as recommended by manufacturer to move door in either direction at minimum << 8 inches (203 mm)>>, maximum << 1 foot (305 mm)>> per second.

Manual Override:  << Not required; or  Crank>>.

Operation Supply Voltage:  << 110V; or  240V>>.

Actuation:  Double throw, hard-wired wall switch.

Motor Mounting:  Within counterbalance tube.

PART 3 EXECUTION

Examination

Verify << existing conditions; and  _______>> meet manufacturer's requirements before starting work.

Verify opening sizes, tolerances and conditions are acceptable.

Installation

Install in accordance with manufacturer's written instructions.

Use anchorage devices to securely fasten assembly to wall construction and building structure without distortion or stress.

Securely and rigidly brace components suspended from structure.<<  Secure guides to structural members only.;    ______ .; or  None - N/A>>

Fit and align assembly including hardware; level and plumb, to provide smooth operation.

Coordinate installation of electrical service; see Section 260583.

Install enclosure and perimeter trim.

TOLERANCES

Maintain dimensional tolerances and alignment with adjacent work.

Maximum Variation from Plumb:  << 1/16 inch (1.6 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.6 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch per 10 feet (3.2 mm per 3 m); or  ____ inch per 10 feet (____ mm per 3 m)>>.

adjusting

Adjust operating assemblies for smooth and noiseless operation.

CLeaning

See Section 017000 - Execution and Closeout Requirements for additional requirements.

Clean installed components.

Remove labels and visible markings.

PROTECTION

Protect finishes until completion of project.

Touch up damaged finishes after Substantial Completion.

END OF SECTION