Section 083326
UPWARd Coiling Grilles

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward coiling metal grilles.

Springless upward coiling metal grilles.

Advanced performance upward coiling metal grilles.

Related Requirements

Section << 055000 - Metal Fabrications>>:  << Support framing;  Framed opening; or  _____>>.

Section << 062000 - Finish Carpentry>>:  Wood casings and moldings.

Section << 083323 - Overhead Coiling Doors>>.

Section << 087100 - Door Hardware>>:  Cylinder cores and keys.

Section << 260533.13 - Conduit for Electrical Systems>>:  Conduit from electric circuit to operator and from operator to control station.

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Reference Standards

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

UL 508A - Industrial Control Panels; Current Edition, Including All Revisions.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Provide general construction component connections and details << and electrical equipment;  _____; or  None - N/A>>.

Provide documentation showing electrical components are listed and classified for purpose specified by <<ITS (DIR)>>, <<UL (DIR)>>, or testing firm acceptable to authorities having jurisdiction.

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Manufacturer's qualification statement.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Project Record Documents:  Include as-built electrical diagrams for electrical operation and connection to fire alarm system.

Quality Assurance

Manufacturer Qualifications:  Company specializing in manufacturing products specified in this section with minimum << three; or  _______>> years of << documented;  _____; or  None - N/A>> experience.

Installer Qualifications:  Company specializing in performing work of type specified and with minimum << three; or  _____>> years<<  documented;    _____; or  None - N/A>> experience<< and approved by manufacturer; or  None - N/A>>.

Delivery, Storage, and Handling

Store products in manufacturer's unopened packaging until ready for installation.

Protect materials from exposure to moisture.

Store materials in dry, warm, ventilated, weathertight location.

Warranty

Manufacturer Door-Only Warranty:  Provide manufacturer warranty for counterbalance shaft assembly for years indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

Manufacturer Door and Operator Warranty:  Manufacturer's limited warranty for door and operator system, except counterbalance spring and finish, free from defects in material and workmanship for years and cycles indicated under individual doors.

PART 2 PRODUCTS

Manufacturer

Overhead Door Corporation; www.overheaddoor.com; 1 (800) 929-3667.

Substitutions: Not permitted.

Upward Coiling Metal Grilles

Overhead DoorTM Brand; Model 670 Aluminum Security Grille.

Grille:  Horizontal bar curtain, coiling on overhead counterbalanced shaft.

~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 30 feet (9144 mm).

For larger grilles, contact manufacturer.

~~~ END OF MASTER NOTE ~~~~

Width:  << _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 17 feet (5182 mm).

- For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _______>>.

Mounting:  << Surface-mounted on side indicated on drawings; or  Between jamb>>.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm>> per second.

Closing Speed:  Door to operate at variable speed << 6 to 12 inches (152 to 305 mm>> per second.

Operation Cycles:  Capable of operating for 20,000 cycles, minimum. One operation cycle is complete when grille is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door-Only Warranty:  One year.

Grille Material:  Aluminum.

Pattern:  << Brick;  Straight lattice;  Straight lattice with polycarbonate inserts; or  _______>>.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Mill finish;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _______; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Spacing options per grille pattern:

Straight: 2 inches (51 mm) on center or 1-1/2 inches (38 mm) on center
Brick: 2 inches (51 mm) on center
Straight with polycarbonate insert: 3 inches (76 mm) on center
~~~ END OF MASTER NOTE ~~~~

Vertical Rod Spacing:  << 1-1/2 inches (38 mm);  2 inches (51 mm);  3 inches (76 mm); or  _____ inches (_____ mm)>> on center.

- ~~~~~ MASTER NOTE ~~~~~~
Spacing options per grille pattern:

Straight: 3 inches (76 mm) on center, 6 inches (152 mm) on center, or 9 inches (228 mm) on center
Brick: 9 inches (228 mm) on center
Straight with polycarbonate inserts: 12 inches (305 mm) on center
~~~ END OF MASTER NOTE ~~~~

Horizontal Link Spacing:  << 3 inches (76 mm);  6 inches (152 mm);  9 inches (228 mm);  12 inches (305 mm); or  _____ inches (_____ mm)>> on center.

Bottom Bar:  << 304 Stainless steel;  6063-T6 aluminum;  Galvanized steel; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Profile options per material:

Aluminum: Extruded T, extruded tube, or double angle
Steel: Single angle, double angle
Stainless Steel: Double angle
~~~ END OF MASTER NOTE ~~~~

Profile:  << Extruded T;  Extruded tube;  Single angle;  Double angle; or  _______>>.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Hot-dip galvanized;  Mill finish;  No.2B mill;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Thumb turns only available with extruded tube bottom bar.
~~~ END OF MASTER NOTE ~~~~

Locking options to include:  << Best cylinder.;  Best cylinder with thumb turn.;  Cylinder.;  Cylinder with thumb turn.;  Slide lock.;  _______; or  None - N/A>>

- ~~~~~ MASTER NOTE ~~~~~~
Center lock location only available with extruded tube bottom bar.
~~~ END OF MASTER NOTE ~~~~

Lock Locations:  << Both jambs;  Center;  Left only;  Right only; or  _______>>.

Lock Access:  << Both sides;  Coil side;  Opposite coil side; or  _______>>.

Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom seal astragal comes standard.
Select "Not required" if adding a sensing edge.
~~~ END OF MASTER NOTE ~~~~

Bottom Seal Astragal:  << Not required;  Required; or  _______>>.

Side Guides, Channels:  Constructed with members fully bolted together.

Material:  << Aluminum;  Steel;  304 Stainless steel; or  _______>>.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Hot-dip galvanized;  Mill finish;  No.2B mill;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

Brackets:  << Stainless steel;  Steel; or  _______>> to support counterbalance and curtain.

- ~~~~~ MASTER NOTE ~~~~~~
PowderGuard Premium powder coat in black is standard finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Hot-dip galvanized;  PowderGuard Premium powder coat; color: black;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat; color:  _______;  PowderGuard Textured powder coat; color:  _______;  PowderGuard Zinc-Enriched powder coat; color:  _______; or  _______>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (2.5 mm per m)>> of span. Adjustable spring tension required.

Hood:  << Aluminum;  Not required;  Steel;  304 Stainless steel; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
304 Stainless steel hood only available in No. 4 finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Galvanized, G90 (Z275);  Match curtain;  Mill finish;  No.4 satin;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat; color:  _______;  PowderGuard Textured powder coat; color:  _______; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 135 sq ft (12.5 sq m) for heights over 10 feet (3048 mm). Chain hoist over 135 sq ft (12.5 sq m) for heights over 10 feet (3048 mm) but less than 15 feet (4572 mm).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Chain hoist;  Crank;  Push-up; or  _____>>.

Motor Operation:  << Not required;  Provide manufacturer’s recommended electric operator; or  _____>>.

Operator:  << UL-listed; and  _____>>.

Size:  As recommended by manufacturer to move door at opening and closing speeds indicated above.

Manual Override:  << Hoist;  Hoist and jackshaft release;  Jackshaft release; or  _____>>.

Timer to Close:  << Automatic closing, controlled by adjustable, hold-open time delay;  Not required; or  _____>>.

Operation Supply Voltage:  << 220V, 1-phase, 50Hz;  220V, 3-phase, 50Hz;  400V, 3-phase, 50Hz;  115/208/230V, 1-phase, 60Hz;  208/230/460V, 3-phase, 60Hz;  575V, 3-phase, 60Hz; or  _____>>.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  _____>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch;  radio control; and  _____>>.

Motor Mounting:  << Bench mount;  Front of hood;  Through wall;  Top of hood;  Wall-mounted; or  _____>>, << right-hand side;  left-hand side; or  _____>>.

Obstruction Safety Detection:

Constant Contact Button:  << No entrapment;  Not required;  Required; or  _____>>.

Photo Eyes:  << NEMA 4X.;  Reflective.;  Standard.;  _____ .; or  None - N/A>>

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Sensing Edge:  << Two-wire reversing;  Wireless reversing; or  _____>>.

Overhead DoorTM Brand; Model 670 Aluminum Security Grille with Emergency Egress Motorized System.

Grille:  Horizontal bar curtain, coiling on overhead counterbalanced shaft.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 30 feet (9144 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 11 feet (3353 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet (_____ mm)>>.

Mounting:  << Between jamb;  Surface-mounted on side indicated on drawings; or  _______>>.

Opening Speed:  Door to operate at variable speed of << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for 20,000 cycles, minimum. One operation cycle is complete when grille is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Grille Material:  Aluminum.

Pattern:  Straight lattice.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Mill finish;  PowderGuard Premium powder  coat to match black anodized;  PowderGuard Premium powder  coat to match bronze anodized;  PowderGuard Premium powder  coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____; or  _____>>.

Vertical Rod Spacing:  << 2 inches (51 mm)>> on center.

Horizontal Link Spacing:  << 3 inches (76 mm);  6 inches (152 mm);  9 inches (228 mm); or  _____ inches (_____ mm)>> on center.

Bottom Bar:  6063-T6 aluminum.

- ~~~~~ MASTER NOTE ~~~~~~
Aluminum: Double angle, extruded T, extruded tube
~~~ END OF MASTER NOTE ~~~~

Profile:  Extruded tube.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Mill finish;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat; color:  _____; or  _____>>.

Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom seal astragal comes standard.
Select "Not required" if adding a sensing edge.
~~~ END OF MASTER NOTE ~~~~

Bottom Seal Astragal:  << Not required;  Required; or  _______>>.

Side Guides, Channels:  Constructed of aluminum with members fully bolted together.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Mill finish;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____; or  _____>>.

Brackets:  << Stainless steel;  Steel; or  _______>> to support counterbalance and curtain.

- ~~~~~ MASTER NOTE ~~~~~~
PowderGuard Premium powder coat in black is standard finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Hot-dip galvanized;  PowderGuard Premium powder coat; color: black;  PowderGuard Zinc-Enriched base powder coat;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder  coat to match bronze anodized;  PowderGuard Premium powder  coat to match black anodized;  PowderGuard Premium powder coat; color:  _______;  PowderGuard Textured powder coat; color:  _______; or  _______>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (2.5 mm per m)>> of span. Adjustable spring tension required.

Hood:  << Aluminum;  Not required;  Steel;  304 Stainless steel; or  _______>>.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Galvanized, G90 (Z275);  Mill finish;  Match curtain;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____; or  _____>>.

Emergency Egress:  Required.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction, minimum << 8 inches (203 mm)>> and maximum << 12 inches (305 mm)>> per second.

Manual Override:  Jackshaft release.

Timer to Close:  << Automatic closing, controlled by adjustable, hold-open time delay;  Not required; or  _______>>.

Operation Supply Voltage:  << 115/208/230V, 1-phase, 60Hz;  208/230/460V, 3-phase, 60Hz;  575V, 3-phase, 60Hz; or  _______>>.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  _______>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch;  radio control; and  _______>>.

Motor Mounting:  << Bench mount;  Front of hood;  Top of hood;  Wall-mounted; or  _______>>, << right-hand side;  left-hand side; or  _______>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor;  Totally enclosed, nonventilated; or  _______>>.

Obstruction Safety Detection:

Constant Contact Button:  << No entrapment;  Not required;  Required; or  _____>>.

Photo Eyes:  << NEMA 4X.;  Reflective.;  Standard.;  _____ .; or  None - N/A>>

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Sensing Edge:  << Two-wire reversing;  Wireless reversing; or  _____>>.

Overhead DoorTM Brand; Model 671 Steel Security Grille.

Grille:  Horizontal bar curtain, coiling on overhead counterbalanced shaft.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 30 feet 4 inches (9244 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 40 feet (12192 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet (_____ mm)>>.

Mounting:  << Between jamb;  Surface-mounted on side indicated on drawings; or  _______>>.

Opening Speed:  Door to operate at variable speed of << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for 20,000 cycles, minimum. One operation cycle is complete when grille is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door-Only Warranty:  One year.

Grille Material:  << Stainless steel;  Steel; or  _______>>.

Pattern:  << Brick;  Straight lattice;  Straight lattice with polycarbonate inserts; or  _______>>.

Finish:  << Galvanized;  No.2 mill finish;  No.4 satin finish;  PowderGuard Premium powder coat; color:  _____; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Spacing options per grille pattern:

Straight: 2 inches (51 mm) on center or 1-1/2 inches (38 mm) on center
Brick: 2 inches (51 mm) on center
Straight with polycarbonate insert: 3 inches (76 mm) on center
~~~ END OF MASTER NOTE ~~~~

Vertical Rod Spacing:  << 1-1/2 inches (38 mm);  2 inches (51 mm);  3 inches (76 mm); or  _____ inches (_____ mm)>> on center.

- ~~~~~ MASTER NOTE ~~~~~~
Spacing options per grille pattern:

Straight: 3 inches (76 mm) on center, 6 inches (152 mm) on center, or 9 inches (228 mm) on center
Brick: 9 inches (228 mm) on center
Straight with polycarbonate inserts: 12 inches (305 mm) on center
~~~ END OF MASTER NOTE ~~~~

Horizontal Link Spacing:  << 3 inches (76 mm);  6 inches (152 mm);  9 inches (228 mm);  12 inches (305 mm); or  _____ inch (_____ mm)>> on center.

Bottom Bar:  << 304 Stainless steel;  6063-T6 aluminum;  Galvanized steel; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Profile options per material:

Aluminum: Extruded T, extruded tube, or double angle
Steel: Single angle, double angle
Stainless Steel: Double angle
~~~ END OF MASTER NOTE ~~~~

Profile:  << Extruded T;  Extruded tube;  Single angle;  Double angle; or  _______>>.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Hot-dip galvanized;  Mill finish;  No.2B mill;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Thumb turns only available with extruded tube bottom bar.
~~~ END OF MASTER NOTE ~~~~

Locking options to include:  << Best cylinder.;  Best cylinder with thumb turn.;  Cylinder.;  Cylinder with thumb turn.;  Slide lock.;  _______ .; or  None - N/A>>

- ~~~~~ MASTER NOTE ~~~~~~
Center lock location only available with extruded tube bottom bar.
~~~ END OF MASTER NOTE ~~~~

Lock Locations:  << Both jambs;  Center;  Left only;  Right only; or  _______>>.

Lock Access:  << Both sides;  Coil side;  Opposite coil side; or  _______>>.

Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom seal astragal comes standard.
Select "Not required" if adding a sensing edge.
~~~ END OF MASTER NOTE ~~~~

Bottom Seal Astragal:  << Not required;  Required; or  _______>>.

Side Guides, Channels:  Constructed with members fully bolted together.

Material:  << Aluminum;  Steel;  304 Stainless steel; or  _______>>.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Hot-dip galvanized;  Mill finish;  No.2B mill;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  black;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

Brackets:  << Stainless steel;  Steel; or  _______>> to support counterbalance and curtain.

- ~~~~~ MASTER NOTE ~~~~~~
PowderGuard Premium powder coat in black is standard finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Hot-dip galvanized;  PowderGuard Premium powder coat; color: black;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat; color:  _______;  PowderGuard Textured powder coat; color:  _______;  PowderGuard Zinc-Enriched powder coat; color:  _______; or  _______>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (2.5 mm per m)>> of span. Adjustable spring tension required.

Hood:  << Aluminum;  Not required;  Steel;  304 Stainless steel; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
304 Stainless steel hood only available in No. 4 finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Black, anodized;  Black, primed;  Bronze, anodized;  Clear, anodized;  Galvanized, G90 (Z275);  Match curtain;  Mill finish;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 135 sq ft (12.5 sq m) for heights over 10 feet (3048 mm).  Chain hoist over 135 sq ft (12.5 sq m) for heights over 10 feet (3048 mm) but less than 15 feet (4572 mm).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Chain hoist;  Crank;  Push-up; or  _______>>.

Motor Operation:  << Not required;  Provide manufacturer’s recommended electric operator; or  _______>>.

Operator:  << UL-listed; and  _______>>.

Size:  As recommended by manufacturer to move door at opening and closing speeds indicated above.

Manual Override:  << Hoist;  Hoist and jackshaft release;  Jackshaft release; or  _______>>.

Timer to Close:  << Automatic closing, controlled by adjustable hold-open time delay;  Not required; or  _______>>.

Operation Supply Voltage:  << 115/208/230V, 1-phase, 60Hz;  208/230/460V, 3-phase, 60Hz;  575V, 3-phase, 60Hz;  220V, 1-phase, 50Hz;  220V, 3-phase, 50Hz;  400V, 3-phase, 50Hz; or  _______>>.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  _______>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch;  radio control; and  _______>>.

Motor Mounting:  << Bench mount;  Front of hood;  Through wall;  Top of hood;  Wall-mounted; or  _______>>, << right-hand side;  left-hand side; or  _______>>.

Obstruction Safety Detection:

Constant Contact Button:  << No entrapment;  Not required;  Required; or  _____>>.

Photo Eyes:  << NEMA 4X.;  Reflective.;  Standard.;  _____ .; or  None - N/A>>

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Sensing Edge:  << Two-wire reversing;  Wireless reversing; or  _____>>.

Overhead DoorTM Brand; Model 671 Steel Security Grille with Emergency Egress Motorized System.

Grille:  Horizontal bar curtain, coiling on overhead counterbalanced shaft.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of __ feet (__ mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of __ feet (__ mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet (_____ mm)>>.

Mounting:  << Between jamb;  Surface-mounted on side indicated on drawings; or  _______>>.

Opening Speed:  Door to operate at variable speed of << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for 20,000 cycles, minimum. One operation cycle is complete when grille is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door-Only Warranty:  One year.

Grille Material:  << Stainless steel;  Steel; or  _______>>.

Pattern:  Straight lattice.

Finish:  << Galvanized;  No.2 mill finish;  No.4 satin finish;  PowderGuard Premium powder coat; color:  _____; or  _____>>.

Vertical Rod Spacing:  << 2 inches (51 mm)>> on center.

Horizontal Link Spacing:  << 3 inches (76 mm);  6 inches (152 mm);  9 inches (228 mm); or  _____ inch (_____ mm)>> on center.

Bottom Bar:  6063-T6 aluminum.

Profile:  Extruded tube.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Hot-dip galvanized;  Mill finish;  No.2B mill;  No.4 satin;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom seal astragal comes standard.
Select "Not required" if adding a sensing edge.
~~~ END OF MASTER NOTE ~~~~

Bottom Seal Astragal:  << Required;  Not required; or  _______>>.

Side Guides, Channels:  Constructed of aluminum with members fully bolted together.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Mill finish;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____; or  _____>>.

Brackets:  << Stainless steel;  Steel; or  _______>> to support counterbalance and curtain.

- ~~~~~ MASTER NOTE ~~~~~~
PowderGuard Premium powder coat in black is standard finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Hot-dip galvanized;  PowderGuard Premium powder coat; color: black;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (2.5 mm per m)>> of span. Adjustable spring tension required.

Hood:  << Aluminum;  Not required;  Steel;  304 Stainless steel; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
304 Stainless steel hood only available in No. 4 finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Galvanized, G90 (Z275);  Match curtain;  Mill finish;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____; or  _____>>.

Emergency Egress:  Required.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction, minimum << 8 inches (203 mm)>> and maximum << 12 inches (305 mm)>> per second.

Manual Override:  Jackshaft release.

Timer to Close:  << Automatic closing controlled by adjustable hold open time delay;  Not required; or  _______>>.

Operation Supply Voltage:  << 115/208/230V, 1-phase, 60Hz;  208/230/460V, 3-phase, 60Hz;  575V, 3-phase, 60Hz; or  _______>>.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  _____>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch;  radio control; and  _______>>.

Motor Mounting:  << Bench mount;  Front of hood;  Top of hood;  Wall-mounted; or  _______>>, << right-hand side;  left-hand side; or  _______>>.

Motor Enclosure:  << NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Not required;  Totally enclosed, fan-cooled motor;  Totally enclosed, non-ventilated; or  _______>>.

Obstruction Safety Detection:

Constant Contact Button:  << No entrapment;  Not required;  Required; or  _____>>.

Photo Eyes:  << NEMA 4X;  Reflective;  Standard;  _____; or  None - N/A>>.

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Sensing Edge:  << Two-wire reversing;  Wireless reversing; or  _____>>.

Overhead DoorTM Brand; Model 674 Perforated Aluminum Panel Security Grille.

Grille:  Horizontal bar curtain, coiling on overhead counterbalanced shaft.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 24 feet 4 inches (7416 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 17 feet 4 inches (5282 mm). For larger grilles, contact manufacturer.  
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet (_____ mm)>>.

Mounting:  << Between jamb;  Surface-mounted on side indicated on drawings; or  _______>>.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed << 6 to 12 inches (152 to 305 mm)>> per second.

Operation Cycles:  Capable of operating for 20,000 cycles, minimum. One operation cycle is complete when grille is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door-Only Warranty:  One year.

Grille Material:  << Aluminum; or  _____>>.

Pattern:  Perforated panel.

Finish:  << Clear anodized;  PowderGuard Premium powder coat; color:  _____; or  _____>>.

Bottom Bar:  6063-T6 aluminum.

Profile:  Extruded tube.

Finish:  Clear anodized.

- ~~~~~ MASTER NOTE ~~~~~~
Thumb turns only available with extruded tube bottom bar.
~~~ END OF MASTER NOTE ~~~~

Locking options to include:  << Best cylinder.;  Best cylinder with thumb turn.;  Cylinder.;  Cylinder with thumb turn.;  Slide lock.;  _______ .; or  None - N/A>>

- ~~~~~ MASTER NOTE ~~~~~~
Center lock location only available with extruded tube bottom bar.
~~~ END OF MASTER NOTE ~~~~

Lock Locations:  << Both jambs;  Center;  Left only;  Right only; or  _______>>.

Lock Access:  << Both sides;  Coil side;  Opposite coil side; or  _______>>.

Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom seal astragal comes standard.
Select "Not required" if adding a sensing edge.
~~~ END OF MASTER NOTE ~~~~

Bottom Seal Astragal:  << Not required;  Required; or  _______>>.

Side Guides, Channels:  Constructed of aluminum with members fully bolted together.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Mill finish;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____; or  _____>>.

Brackets:  Galvanized steel to support counterbalance and curtain.

Finish:  << Galvanized;  PowderGuard Premium powder coat; color:  _______; or  _______>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (2.5 mm per m)>> of span. Adjustable spring tension required.

Hood:  << Aluminum;  Not required;  Steel; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
304 Stainless steel hood only available in No. 4 finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Match curtain;  Mill finish;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 96 sq ft (8.9 sq m) or for heights over 10 feet (3048 mm).  Chain hoist over 96 sq ft (8.9 sq m) for heights over 10 feet (3048 mm) but less than 15 feet (4572 mm).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Chain hoist;  Crank;  Push-up; or  _______>>.

Motor Operation:  << Not required;  Provide manufacturer’s recommended electric operator; or  _______>>.

Operator:  << UL-listed; and  _______>>.

Size:  As recommended by manufacturer to move door at opening and closing speeds indicated above.

Manual Override:  << Hoist;  Hoist and jackshaft release;  Jackshaft release; or  _______>>.

Timer to Close:  << Automatic closing controlled by adjustable hold open time delay;  Not required; or  _______>>.

Operation Supply Voltage:  << 115/208/230V, 1-phase, 60Hz;  208/230/460V, 3-phase, 60Hz;  575V, 3-phase, 60Hz;  220V, 1-phase, 50Hz;  220V, 3-phase, 50Hz;  400V, 3-phase, 50Hz; or  _______>>.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  _______>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch;  radio control; and  _______>>.

Motor Mounting:  << Bench mount;  Front of hood;  Through wall;  Top of hood;  Wall-mounted; or  _______>>, << right-hand side;  left-hand side; or  _______>>.

Obstruction Safety Detection:

Constant Contact Button:  << No entrapment;  Not required;  Required; or  _____>>.

Photo Eyes:  << NEMA 4X.;  Reflective.;  Standard.;  _____ .; or  None - N/A>>

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Sensing Edge:  << Two-wire reversing;  Wireless reversing; or  _____>>.

Springless Upward Coiling Metal Grilles

Overhead DoorTM Brand; Model 671S.

Grille:  Horizontal bar curtain, coiling on overhead counterbalanced shaft.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 30 feet (9144 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 14 feet 4 inches (4369 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet (_____ mm)>>.

Mounting:  Surface-mounted on side indicated on drawings.

Opening Speed:  Door to operate at variable speed of << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for 300,000 cycles, minimum. One operation cycle is complete when grille is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Limited Warranty:  Two years.

Grille Material:

Grille Rod:  << Stainless steel;  Steel; or  _______>>.

Grille Spacer Tubes:  << Aluminum;  304 Stainless steel; or  _______>>.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Mill finish;  No.2B mill;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____; or  _____>>.

Grille Links:  << Aluminum;  Stainless steel;  Steel; or  _______>>.

Finish:  << Galvanized;  Mill finish;  No.2B mill;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____; or  _____>>.

Pattern:  << Brick;  Straight lattice; or  _______>>.

Vertical Rod Spacing:  << 2 inches (51 mm)>>.

Horizontal Link Spacing:  << 9 inches (229 mm)>>.

Bottom Bar:  << 6063-T6 aluminum;  Galvanized steel; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Profile options per material:

Aluminum: Extruded tube or double angle
Steel: Double angle
~~~ END OF MASTER NOTE ~~~~

Profile:  << Double angle;  Extruded tube; or  _______>>.

Finish:  << Black anodized;  Bronze anodized;  Clear anodized;  Hot-dip galvanized;  Mill finish;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _______; or  _____>>.

Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom seal astragal comes standard.
Select "Not required" if adding a sensing edge.
~~~ END OF MASTER NOTE ~~~~

Bottom Seal Astragal:  << Required;  Not required; or  _______>>.

Side Guides, Channels:  Constructed of steel with members fully bolted together. Three-angle structural steel high usage guide.

- ~~~~~ MASTER NOTE ~~~~~~
PowderGuard Premium powder coat in black is standard finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Hot-dip galvanized;  PowderGuard Premium powder coat; color: black;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

Brackets:  Steel to support counterbalance and curtain.

- ~~~~~ MASTER NOTE ~~~~~~
PowderGuard Premium powder coat in black is standard finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Hot-dip galvanized;  PowderGuard Premium powder coat; color: black;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

Door Roll:  Directly driven, steel, springless roll with integral shafts, keyed on drive end and supported by self-aligning greaseable sealed bearings. Counterbalance device not required.

Hood:  << Aluminum;  Not required;  Galvanized steel;  Stainless steel; or  _______>>.

Finish:  << Black, primed;  Galvanized, G90 (Z275);  Match curtain;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____; or  _____>>.

Motor Operation:  Direct drive, integrated gear motor and brake assembly, sized as recommended by manufacturer to move door in either direction, minimum << 8 inches (203 mm)>> and maximum << 12 inches (305 mm)>> per second. Operator and drive assembly is factory pre-assembled and provided with all wiring harnesses needed direct from factory.

Manual Override:  << Hoist;  Hoist and jackshaft release;  Jackshaft release; or  _______>>.

Timer to Close:  Time delay self-close timer and nonresettable cycle counter.

Operation Supply Voltage:  << 115/208/230V, 1-phase, 60Hz;  208/230/460V, 3-phase, 60Hz;  575V, 3-phase, 60Hz;  220V, 1-phase, 50Hz;  220V, 3-phase, 50Hz;  400V, 3-phase, 50Hz; or  _______>>.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  _______>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch;  radio control; and  _______>>.

Motor Mounting:  << Front of hood;  Wall-mounted; or  _______>>, << right-hand side;  left-hand side; or  _______>>.

Obstruction Safety Detection:

Primary, Monitored:  << Dual photo eyes;  Reflective photo eyes;  Thru-beam photo eyes;  Two-wire reversing safety edge; or  _______>>.

Secondary, Unmonitored:  << Dual photo eyes;  Reflective photo eyes;  Thru-beam photo eyes;  Two-wire reversing safety edge; or  _______>>.

Control Panel:  Electronic controller with microprocessor self-diagnostics. Digital readout indicates door action, alarm conditions, and fault conditions. Includes time delay self-close timer and nonresettable cycle counter. Enclosure is IP54-rated, NEMA 3 equivalent.

Advanced Performance Upward Coiling Metal Grilles

Overhead DoorTM Brand; Model 676.

Grille:  Horizontal bar curtain, coiling on overhead counterbalanced shaft.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 30 feet 4 inches (9246 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 14 feet 4 inches (4369 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet (_____ mm)>>.

Mounting:  Surface-mounted on side indicated on drawings.

Opening Speed:  Door to operate at variable speed of << 24 inches per second (610 mm per second)>>.

Closing Speed:  Door to operate at variable speed of << 12 inches per second (305 mm per second)>>.

Operation Cycles:  Capable of operating for 300,000 cycles, minimum. One operation cycle is complete when grille is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Limited Warranty:  Two years.

Grille Material:

Grille Rod:  << Stainless steel;  Steel; or  _______>>.

Grille Spacer Tubes:  << Aluminum;  304 Stainless steel; or  _______>>.

Finish:  << Black anodized;  Bronze anodized;  Clear anodized;  Mill finish;  No.2B mill;  No.4 satin;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat; color:  ___; or  _______>>.

Grille Links:  << Aluminum;  Stainless steel;  Steel; or  _______>>.

Finish:  << Galvanized;  Mill finish;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____; or  _____>>.

Pattern:  << Brick;  Straight lattice; or  _______>>.

Vertical Rod Spacing:  << 2 inches (51 mm)>>.

Horizontal Link Spacing:  << 9 inches (229 mm)>>.

Bottom Bar:  << 6063-T6 aluminum;  Galvanized steel; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Profile options per material:

Aluminum: Extruded tube or double angle
Steel: Double angle
~~~ END OF MASTER NOTE ~~~~

Profile:  << Extruded tube;  Double angle; or  _______>>.

Finish:  << Black anodized;  Bronze anodized;  Clear anodized;  Hot-dip galvanized;  Mill finish;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom seal astragal comes standard.
Select "Not required" if adding a sensing edge.
~~~ END OF MASTER NOTE ~~~~

Bottom Seal Astragal:  << Required;  Not required; or  _______>>.

Side Guides, Channels:  Constructed of steel with members fully bolted together. Three-angle structural steel high usage guide.

Finish:  << Hot-dip galvanized;  PowderGuard Premium powder coat; color: black;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

Brackets:  Galvanized steel to support counterbalance and curtain.

Finish:  << Hot-dip galvanized;  PowderGuard Premium powder coat; color: black;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat; color:  _______;  PowderGuard Textured powder coat; color:  _______;  PowderGuard Zinc-Enriched powder coat; color:  _______; or  _______>>.

Door Roll:  Directly driven, steel, springless roll with integral shafts, keyed on drive end and supported by self-aligning greaseable sealed bearings. Counterbalance device not required.

Hood:  << Aluminum;  Not required;  Galvanized steel;  Stainless steel; or  _______>>.

Finish:  << Black, primed;  Galvanized, G90 (Z275);  Match curtain;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat; color:  _______;  PowderGuard Textured powder coat; color:  _______; or  _______>>.

Motor Operation:  Direct drive, integrated gear motor and brake assembly, sized as recommended by manufacturer to move door in either direction, minimum << 8 inches (203 mm)>> and maximum << 12 inches (305 mm)>> per second. Operator and drive assembly is factory pre-assembled and provided with all wiring harnesses needed direct from factory.

Manual Override:  Hoist or crank, determined by motor recommended by manufacturer.

Timer to Close:  Time delay self-close timer and nonresettable cycle counter.

Operation Supply Voltage:  << 208V, 1-phase, 60Hz;  208V, 3-phase, 60Hz;  575V, 3-phase, 60Hz;  220-240V, 1-phase, 50Hz;  220V, 3-phase, 50Hz;  440-480V, 3-phase, 50Hz; or  _______>>.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  _______>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch;  radio control; and  _______>>.

Motor Mounting:  << Front of hood;  Wall-mounted; or  _______>>, << right-hand side;  left-hand side; or  _______>>.

Obstruction Safety Detection:

Primary, Monitored:  Wireless reversing safety edge.

Secondary, Unmonitored:  << Dual photo eyes;  Dual thru-beam photo eyes;  Reflective photo eyes;  Thru-beam photo eyes; or  _______>>.

Control Panel:  Variable frequency drive controller with microprocessor self-diagnostics. LCD readout indicates door action, alarm conditions, and fault conditions. Includes timer to close programming options and nonresettable cycle counter. Enclosure is NEMA 4X-rated. Control system in accordance with <<UL 508A>>. Junction box is IP67-rated.

PART 3 EXECUTION

Examination

Verify adjacent construction is suitable for grille installation.

Verify electrical services are installed and accessible.

Verify grille opening is plumb, header is level, and dimensions are correct.

Notify <<Architect>> of unacceptable conditions or varying dimensions.

Commencing installation indicates acceptance of substrate and grille opening conditions.

Installation

Install grille unit assembly in accordance with manufacturer's instructions.

Securely fasten assembly to wall construction using anchorage devices and building framing without distortion or stress.

Securely and rigidly brace components suspended from structure.<<  Secure guides to structural members only.;    _____ .; or  None - N/A>>

Fit and align assembly including hardware, level and plumb, to provide smooth operation.

See Section << 260583>> to coordinate installation of electrical service.

Complete wiring from disconnect to unit components.

Install enclosure and perimeter trim.

Tolerances

Maintain dimensional tolerances and alignment with adjacent work.

Maximum Variation From Plumb:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Maximum Variation From Level:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch per 10 feet (3 mm per 3 m); or  ____ inch per 10 feet (__ mm per 3 m)>> straight edge.

Adjusting

Adjust grille, hardware, and operating assemblies for smooth and noiseless operation.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean grille and components.

Remove labels and visible markings.

- END OF SECTION