Section 083326
UPWARd Coiling Grilles

Model 671

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward coiling metal grilles.

Related Requirements

Section << 055000 - Metal Fabrications>>:  << Support framing;  Framed opening; or  _____>>.

Section << 062000 - Finish Carpentry>>:  Wood casings and moldings.

Section << 083323 - Overhead Coiling Doors>>.

Section << 087100 - Door Hardware>>:  Cylinder cores and keys.

Section << 260533.13 - Conduit for Electrical Systems>>:  Conduit from electric circuit to operator and from operator to control station.

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Reference Standards

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

UL 508A - Industrial Control Panels; Current Edition, Including All Revisions.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Provide general construction component connections and details << and electrical equipment;  _____; or  None - N/A>>.

Provide documentation showing electrical components are listed and classified for purpose specified by <<ITS (DIR)>>, <<UL (DIR)>>, or testing firm acceptable to authorities having jurisdiction.

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Manufacturer's qualification statement.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Quality Assurance

Manufacturer Qualifications:  Company specializing in manufacturing products specified in this section with minimum << three; or  _______>> years of << documented;  _____; or  None - N/A>> experience.

Installer Qualifications:  Company specializing in performing work of type specified and with minimum << three; or  _____>> years<<  documented;    _____; or  None - N/A>> experience<< and approved by manufacturer; or  None - N/A>>.

Delivery, Storage, and Handling

Store products in manufacturer's unopened packaging until ready for installation.

Protect materials from exposure to moisture.

Store materials in dry, warm, ventilated, weathertight location.

Warranty

Manufacturer Door-Only Warranty:  Provide manufacturer warranty for counterbalance shaft assembly for years indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

PART 2 PRODUCTS

Manufacturer

Overhead Door Corporation; www.overheaddoor.com; 1 (800) 929-3667.

Substitutions: Not permitted.

Upward Coiling Metal Grilles

Overhead DoorTM Brand; Model 671 Steel Security Grille.

Grille:  Horizontal bar curtain, coiling on overhead counterbalanced shaft.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 30 feet 4 inches (9244 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet (_____ mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 40 feet (12192 mm).
For larger grilles, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet (_____ mm)>>.

Mounting:  << Between jamb;  Surface-mounted on side indicated on drawings; or  _______>>.

Opening Speed:  Door to operate at variable speed of << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for 20,000 cycles, minimum. One operation cycle is complete when grille is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door-Only Warranty:  One year.

Grille Material:  << Stainless steel;  Steel; or  _______>>.

Pattern:  << Brick;  Straight lattice;  Straight lattice with polycarbonate inserts; or  _______>>.

Finish:  << Galvanized;  No.2 mill finish;  No.4 satin finish;  PowderGuard Premium powder coat; color:  _____; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Spacing options per grille pattern:

Straight: 2 inches (51 mm) on center or 1-1/2 inches (38 mm) on center
Brick: 2 inches (51 mm) on center
Straight with polycarbonate insert: 3 inches (76 mm) on center
~~~ END OF MASTER NOTE ~~~~

Vertical Rod Spacing:  << 1-1/2 inches (38 mm);  2 inches (51 mm);  3 inches (76 mm); or  _____ inches (_____ mm)>> on center.

- ~~~~~ MASTER NOTE ~~~~~~
Spacing options per grille pattern:

Straight: 3 inches (76 mm) on center, 6 inches (152 mm) on center, or 9 inches (228 mm) on center
Brick: 9 inches (228 mm) on center
Straight with polycarbonate inserts: 12 inches (305 mm) on center
~~~ END OF MASTER NOTE ~~~~

Horizontal Link Spacing:  << 3 inches (76 mm);  6 inches (152 mm);  9 inches (228 mm);  12 inches (305 mm); or  _____ inch (_____ mm)>> on center.

Bottom Bar:  << 304 Stainless steel;  6063-T6 aluminum;  Galvanized steel; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Profile options per material:

Aluminum: Extruded T, extruded tube, or double angle
Steel: Single angle, double angle
Stainless Steel: Double angle
~~~ END OF MASTER NOTE ~~~~

Profile:  << Extruded T;  Extruded tube;  Single angle;  Double angle; or  _______>>.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Hot-dip galvanized;  Mill finish;  No.2B mill;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Thumb turns only available with extruded tube bottom bar.
~~~ END OF MASTER NOTE ~~~~

Locking options to include:  << Best cylinder.;  Best cylinder with thumb turn.;  Cylinder.;  Cylinder with thumb turn.;  Slide lock.;  _______ .; or  None - N/A>>

- ~~~~~ MASTER NOTE ~~~~~~
Center lock location only available with extruded tube bottom bar.
~~~ END OF MASTER NOTE ~~~~

Lock Locations:  << Both jambs;  Center;  Left only;  Right only; or  _______>>.

Lock Access:  << Both sides;  Coil side;  Opposite coil side; or  _______>>.

Seals:

- ~~~~~ MASTER NOTE ~~~~~~
Bottom seal astragal comes standard.
Select "Not required" if adding a sensing edge.
~~~ END OF MASTER NOTE ~~~~

Bottom Seal Astragal:  << Not required;  Required; or  _______>>.

Side Guides, Channels:  Constructed with members fully bolted together.

Material:  << Aluminum;  Steel;  304 Stainless steel; or  _______>>.

Finish:  << Black, anodized;  Bronze, anodized;  Clear, anodized;  Hot-dip galvanized;  Mill finish;  No.2B mill;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  black;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____;  PowderGuard Zinc-Enriched powder coat; color:  _____; or  _____>>.

Brackets:  << Stainless steel;  Steel; or  _______>> to support counterbalance and curtain.

- ~~~~~ MASTER NOTE ~~~~~~
PowderGuard Premium powder coat in black is standard finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Hot-dip galvanized;  PowderGuard Premium powder coat; color: black;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat; color:  _______;  PowderGuard Textured powder coat; color:  _______;  PowderGuard Zinc-Enriched powder coat; color:  _______; or  _______>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (2.5 mm per m)>> of span. Adjustable spring tension required.

Hood:  << Aluminum;  Not required;  Steel;  304 Stainless steel; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
304 Stainless steel hood only available in No. 4 finish.
~~~ END OF MASTER NOTE ~~~~

Finish:  << Black, anodized;  Black, primed;  Bronze, anodized;  Clear, anodized;  Galvanized, G90 (Z275);  Match curtain;  Mill finish;  No.4 satin;  PowderGuard Premium powder coat to match black anodized;  PowderGuard Premium powder coat to match bronze anodized;  PowderGuard Premium powder coat to match clear anodized;  PowderGuard Premium powder coat; color:  _____;  PowderGuard Textured powder coat; color:  _____; or  _____>>.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 135 sq ft (12.5 sq m) for heights over 10 feet (3048 mm).  Chain hoist over 135 sq ft (12.5 sq m) for heights over 10 feet (3048 mm) but less than 15 feet (4572 mm).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Chain hoist;  Crank;  Push-up; or  _______>>.

Motor Operation:  << Not required;  Provide manufacturer’s recommended electric operator; or  _______>>.

Operator:  << UL-listed; and  _______>>.

Size:  As recommended by manufacturer to move door at opening and closing speeds indicated above.

Manual Override:  << Hoist;  Hoist and jackshaft release;  Jackshaft release; or  _______>>.

Timer to Close:  << Automatic closing, controlled by adjustable hold-open time delay;  Not required; or  _______>>.

Operation Supply Voltage:  << 115/208/230V, 1-phase, 60Hz;  208/230/460V, 3-phase, 60Hz;  575V, 3-phase, 60Hz;  220V, 1-phase, 50Hz;  220V, 3-phase, 50Hz;  400V, 3-phase, 50Hz; or  _______>>.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  _______>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch;  radio control; and  _______>>.

Motor Mounting:  << Bench mount;  Front of hood;  Through wall;  Top of hood;  Wall-mounted; or  _______>>, << right-hand side;  left-hand side; or  _______>>.

Obstruction Safety Detection:

Constant Contact Button:  << No entrapment;  Not required;  Required; or  _____>>.

Photo Eyes:  << NEMA 4X.;  Reflective.;  Standard.;  _____ .; or  None - N/A>>

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Sensing Edge:  << Two-wire reversing;  Wireless reversing; or  _____>>.

PART 3 EXECUTION

Examination

Verify adjacent construction is suitable for grille installation.

Verify electrical services are installed and accessible.

Verify grille opening is plumb, header is level, and dimensions are correct.

Notify <<Architect>> of unacceptable conditions or varying dimensions.

Commencing installation indicates acceptance of substrate and grille opening conditions.

Installation

Install grille unit assembly in accordance with manufacturer's instructions.

Securely fasten assembly to wall construction using anchorage devices and building framing without distortion or stress.

Securely and rigidly brace components suspended from structure.<<  Secure guides to structural members only.;    _____ .; or  None - N/A>>

Fit and align assembly including hardware, level and plumb, to provide smooth operation.

See Section << 260583>> to coordinate installation of electrical service.

Complete wiring from disconnect to unit components.

Install enclosure and perimeter trim.

Tolerances

Maintain dimensional tolerances and alignment with adjacent work.

Maximum Variation From Plumb:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Maximum Variation From Level:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch per 10 feet (3 mm per 3 m); or  ____ inch per 10 feet (__ mm per 3 m)>> straight edge.

Adjusting

Adjust grille, hardware, and operating assemblies for smooth and noiseless operation.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean grille and components.

Remove labels and visible markings.

- END OF SECTION